### Forensic Building Envelope Analysis: Ventilation Failure & Thermal Tracking

Date: January 10, 2026
Roof install date:  April 12, 2024

Subject: Roof Ventilation Imbalance & Interior Environmental Failure

Property Type: 1,700 sq. ft. Rancher (4/12 Slope), Victoria, BC (Zone 4C)

### 1. Executive Summary

A forensic review of the recently installed roof system identifies a critical failure in the ventilation strategy. The installation of high-volume continuous exhaust (Omni Ridge Vent) paired with restricted, non-compliant intake (Menzies SV-50) has created a state of Attic Depressurization (Negative Pressure Gradient).

This negative pressure is actively pulling conditioned indoor air through the ceiling plane, resulting in "Thermal Tracking" (ghosting) along joist lines. The intake vents currently installed are prohibited by the manufacturer for this specific application, constituting a breach of installation standards.

### 2. Quantitative Analysis: The Flow Imbalance

While the total amount of ventilation may technically meet the minimum square footage requirement of IRC R806.2, the system violates the physics of balanced ventilation. Best practices dictate a 50/50 balance or a 60/40 intake-heavy ratio.

#### A. Exhaust Capacity (The Drive)

- Product: Omni Ridge Vent (Continuous)

- Spec: 18 sq. in. Net Free Area (NFA) per linear foot.

- Quantity: 60 linear feet.

- Total Exhaust NFA: 1$60 \times 18 =$ 1,080 sq. in.2

#### B. Intake Capacity (The Choke)

- Product: Menzies Metal Products SV-50 Plastic Roof Vent.

- Spec: 50 sq. in. NFA per unit (Nominal).

- Quantity: 12 units.

- Theoretical Intake NFA: $12 \times 50 =$ 600 sq. in.

- Real-World Effective NFA: Due to the 4/12 slope and "reverse flow" turbulence (using an exhaust vent for intake), effective capacity is estimated to be reduced by 40-60%.

  - Estimated Effective Intake: ~300 - 360 sq. in.

#### C. The Deficit

- The Ratio: The Exhaust capacity exceeds Intake capacity by a factor of roughly 3:1.

- Status: Severe Depressurization. The ridge vent acts as a vacuum pump. To satisfy the airflow demand, it pulls the missing air volume from the house interior, bypassing the restricted roof vents.

### 3. Product Compliance & Legal Standard

The primary failure point is the misuse of the Menzies SV-50 vent.

- Manufacturer Prohibition:

The Menzies SV-50 Product Manual explicitly states: "Not to be used as intake vent." The internal baffles are designed to shed rain while exhausting air; forcing air inwards creates turbulence and water ingress risks.

- Slope Violation:

The product is designated for "Steep Roof Application" (Minimum 3.5/12 or 4/12 depending on revision). Using a static vent on the eave of a low-slope roof places the vent in the wind boundary layer, where it often experiences neutral pressure, rendering it ineffective as an intake.

Legal Standard: The use of products contrary to manufacturer instructions constitutes a definitive installation failure. Manufacturer prohibitions define the scope of warranty coverage and professional liability; they cannot be overridden by "common practice."

### 4. The Failure Mechanism: "Thermal Tracking"

The sudden appearance of black lines (Ghosting) immediately following the roof installation provides the temporal evidence of failure.

- Pressure Differential: The unbalanced Ridge Vent creates a negative pressure ($\Delta P$) in the attic relative to the living space.

- Infiltration: This suction pulls warm, particulate-laden indoor air through the ceiling insulation (via pot lights, wire penetrations, and top plates).

- Deposition: As this dirty air passes over the cold ceiling joists (Thermal Bridges), the particles (soot/dust) condense and stick to the drywall.3

- Causality: The roof change was the only variable. The home performed for 20 years without ghosting because the previous roof did not induce high-suction negative pressure.

### 5. Anticipated Defenses & Rebuttals

- Defense: "This is common practice; we use these all the time on low slopes."

  - Rebuttal: "Common practice" does not override the Manufacturer's Written Prohibition. Additionally, the immediate onset of ghosting proves that while it may work on leakier houses, it has failed on this specific building envelope.

- Defense: "The ghosting is caused by candles or a dirty house."

  - Rebuttal: The ghosting appeared immediately after the roof installation. Occupant habits (cooking/cleaning) did not change overnight. The only changed variable is the roof pressure system.

- Defense: "The total NFA meets code."

  - Rebuttal: Meeting the total NFA minimum does not excuse the imbalance or the misuse of product. A system that meets code but destroys the interior via depressurization is still a failed system.

### 6. Remediation Requirements

To stop the interior damage and restore building envelope integrity, the following is required:

- Retrofit Intake: Installation of continuous soffit venting (e.g., 3" strip or perforated soffit) to provide >1,080 sq. in. of Intake NFA to match the ridge vent.

- Balance the System: Disconnect or block the SV-50s (or convert them to passive exhaust if intake is handled elsewhere).

- Verification: A temporary "Blockage Test" (sealing the ridge vent with plastic for 2 weeks) can be performed to confirm that stopping the exhaust stops the ghosting accumulation.

### 7. Validation

This diagnosis has been cross-referenced against building science principles regarding Stack Effect, Bernoulli's Principle, and Manufacturer Specifications.

References:

- Menzies SV-50 Specs ("Not for Intake"): Menzies Product Data

- Lomanco Omni Ridge Specs (18 sq in/ft): Lomanco Omni Data

- Ghosting & Air Movement: ServiceMaster Restoration Guide

- IRC Ventilation Code (Balance Ratio): Burgess Construction Consultants Analysis