# Project Status Update: Ventilation Strategy & Verification

Date: January 16, 2026

Property: 1517 San Juan Ave, Victoria

Status: CRITICAL PAUSE (Insulation & Ventilation)

## 1. Executive Summary

The "Vacuum Effect" diagnosis (Attic Depressurization) has been stress-tested against local industry feedback (Parker Johnston), broader industry consensus (Reddit), and expert building science forensics.

- Conclusion: The diagnosis holds up. The house is suffering from a massive pressure imbalance caused by a high-performance exhaust system (Omni Ridge Vent) paired with insufficient intake.

- The Solution: We are proceeding with a "Belt and Suspenders" approach: fixing the seal (Insulation) AND fixing the pressure (Soffit Ventilation).

## 2. Updates & Verification (Jan 16, 2026)

### A. The "Todd Call" (Parker Johnston)

Feedback Received:

- "Common Practice": Todd noted that using SV-50s as intake is standard for Victoria roofers.

  - Analysis: This explains why Mark did it ("Survivor Bias"), but it failed on your house because of the massive 60-foot Ridge Vent.

- The "Short Circuit" Admission: Todd confirmed that if you install continuous soffit vents, the SV-50s would start acting as exhaust vents.

  - Strategic Win: This independently validates Mark’s own text. It proves that to upgrade the house, the SV-50s must be removed.

### B. Independent Industry Verification (Reddit/Building Science)

Research into professional roofing discussions confirmed:

- The Vacuum is Real: Installing Ridge Vent without Soffit Vents creates a vacuum that sucks conditioned air through the ceiling.

- No Mixing Vents: The consensus is "Do Not Mix Exhaust Systems." Having SV-50s (mid-roof) and Ridge Vents (peak) causes short-circuiting.

### C. Expert Intelligence: Joseph Lstiburek (Building Science Corp)

Source: Builder's Guide to Cold Climates (2022 Edition) & Video Lecture "To Vent or Not to Vent"

Key forensic insights that validate our strategy:

- The "Don't Suck" Rule: Lstiburek explicitly states: "You don't want to have lots of holes up top and a few holes down below because you don't want your attic to suck." If exhaust exceeds intake, the attic depressurizes and pulls air from the house.

- The "Wash the Deck" Priority: He confirms that you must "wash the entire underside of the roof deck" with cold air to prevent ice dams. Mid-roof vents (SV-50s) leave the bottom 4-6 feet of the deck unwashed (a "Dead Zone" for mold).

- The Snow Blanket: Even if we insulate the floor, snow acts as insulation (R-2/inch). Without soffit ventilation to flush the heat, the roof deck will warm up and cause condensation, regardless of attic sealing.

### D. The "Hard Law" (Code & Standards Compliance)

We have identified specific violations of the BC Building Code (BCBC) and CSA Standards that force the removal of the SV-50s.

1. The "Airway" Violation (BCBC 9.19.1.3)

- The Law: The code mandates a 63mm (2.5 inch) airspace between the insulation and roof sheathing specifically at the wall plates.

- The Violation: Because the SV-50s are located ~4 feet up the roof, there is zero airflow entering the mandatory 63mm gap at the eaves. You cannot comply with the "Airway" requirement if the intake is located above the airway.

2. The "Infiltration" Violation (CSA A93:19)

- The Law: Vents must be tested for weather infiltration (rain/snow entry).

- The Violation: Passive vents like the SV-50 are tested to keep rain out while pushing air out (Exhaust). Using them as intake puts them under negative pressure (suction). This creates a "Vacuum Infiltration" risk (sucking in rain/snow) that voids the CSA certification.

## 3. The "Belt & Suspenders" Strategy

We are no longer choosing between "Sealing" and "Ventilation." We are doing both.

| Component | Responsibility | Action | Purpose |
| --- | --- | --- | --- |
| The Suspenders | Advance Insulation | Remove old insulation & Foam Seal attic floor. | Stops the Leak: Prevents moisture from getting into the attic. |
| The Belt | Parker Johnston | Cut Continuous Strip Soffit Vents. | Stops the Vacuum: "Feeds the beast" (Ridge Vent) so it stops pulling on the house. |
| The Correction | Vintage Roofing (Mark) | Remove SV-50 Vents & Patch Deck. | Stops the Short-Circuit: Prevents air from bypassing the new soffits. |

## 4. Immediate Action Plan

### Step 1: Secure the Quote from Todd (Parker Johnston)

Goal: Get the price for the Continuous Strip Vent (Labor Only).

Updated Script for Todd:

"Todd, I've confirmed with my Energy Advisor (using BCBC 9.19.1.3) that we need to establish airflow at the eaves to meet the 63mm airway requirement. The current vents leave a dead zone at the bottom.

I am proceeding with the 'Belt and Suspenders' approach: Advance Insulation will seal the floor, and I need you to cut the continuous strip vent to balance the pressure. Please send over the quote."

### Step 2: The "Olive Branch" Email to Mark

Trigger: Send this after you have the quote from Todd.

Subject: Warranty Claim: Ventilation Correction - 1517 San Juan Ave

Hi Mark,

I am writing to update you on the mold remediation plan. I have paused the insulation work to address the root cause: a severe ventilation imbalance (Vacuum Effect) caused by the roof system installed in April 2024.

I have consulted with independent roofing specialists and an Energy Advisor to verify the physics. I am submitting this Warranty Claim based on manufacturer specifications, the BC Building Code, and your original contract (Quote #203).

1. The Engineering Failure (The Math)

Your Quote #203 guaranteed "Adequate Venting."

- Exhaust: The 60-foot Omni Ridge Vent pulls 1,080 sq. in.

- Intake: The SV-50s provide only 600 sq. in.

- Result: A massive vacuum that sucks warm, dirty air from my living space into the attic (confirmed by Lstiburek/Building Science guidelines).

2. Code Violation (BCBC 9.19.1.3)

The BC Building Code requires a 63mm (2.5 inch) airspace between the insulation and sheathing at the wall plates. Because your SV-50s are located mid-roof, the bottom 4 feet of the roof deck has zero airflow, violating this requirement and creating a mold incubation zone.

3. Product Misuse (CSA A93 & Manufacturer Spec)

Menzies Metal Products confirms the SV-50 is "Not to be used as an intake vent." Furthermore, using an exhaust vent as an intake creates a "Vacuum Infiltration" risk (sucking in rain/snow) that violates CSA A93 weather testing standards.

The Resolution Plan (The "Split Solution")

To resolve this amicably, I will pay for the difficult work (cutting new soffits), saving you that labor.

- My Part: I will hire a siding company to cut continuous soffit vents to finally provide the required intake.

- Your Part (Warranty Repair): You simply need to remove the 12 non-compliant SV-50 vents and patch the roof deck/shingles at your cost.

Next Steps:

Please confirm you agree to this plan so I can coordinate your patch crew with my siding crew.

Regards,

Richard Fremmerlid

1517 San Juan Ave, Victoria