## Project Snapshot

- Home Specs: 1,700 sq. ft. Rancher, Low Slope roof.

- Recent Work (March 2024): New roof by Vintage Roofing (Mark).

  - Installed: Roofnado Stormtread underlayment, 3/8” plywood substrate, BP Signature shingles.

  - Ventilation Installed: 60’ of Omni Ridge Vent (Exhaust) + SV-50 Vents (Intake).

- Current Issues:

  - Moisture: Shadowing/black mold starting on ceiling joists; moisture seen under vapor barrier.

  - Ventilation Imbalance: Massive exhaust (Ridge) vs. questionable intake (SV-50s vs. Stucco).

## The Core Conflicts (Facts vs. Opinions)

#### 1. The "SV-50" Intake Debate

- The Issue: Vintage Roofing installed SV-50s low on the roof for intake.

- The Conflict:

  - Written Specs (The "No"): Manufacturer data sheet and QAI Listing B1024-2 state: "Minimum 3.5:12 Pitch" and "Do not use as intake vent".

  - Verbal Advice (The "Yes"): Menzies Nanaimo Manager (20 years exp) and local installers say it is "standard practice" and "works fine".

- Your Risk: On a low slope roof, these vents are prone to scooping water/snow if used for intake, regardless of what the rep says on the phone.

#### 2. The Quantity Discrepancy

- Notes say: "Installed 12-air intake vents using SV-50".

- Image/Quote says: "Installing 8-air intake vents".

- Action: Verify exactly how many holes are currently cut in your roof.

#### 3. The Ventilation Imbalance

- Exhaust: 60 ft of Omni Vent = ~1,080 sq. in. of exhaust (massive vacuum).

- Intake: Even with 12 SV-50s (600 sq in) + 2 soffit vents, you have less intake than exhaust.

- Result: The ridge vent may be sucking moldy air from your house into the attic.

### 📝 Action Plan & Checklist

#### Phase 1: Immediate Decisions (Before Insulation)

- Resolve SV-50s: Decide whether to trust the written specs (remove/seal them) or the verbal advice (keep them). Recommendation: If keeping them, demand a written letter of indemnity from the roofer.

- Soffit Decision:

  - Option A: Cut 4-inch Pucks (easier, less airflow).

  - Option B: Cut Continuous Strip (Justin's advice, better airflow, harder to find installer).

- Testing: Book EnerTech for the Smoke/Blower Door test to find the air leaks causing the mold before you cover them with R-50.  (decided this wouldn’t help when I talked to him)

#### Phase 2: The Insulation Job (Scope of Work)

- [ ] Removal: Vacuum out old cellulose.

- [ ] Air Sealing: Foam seal ALL penetrations (wires, fans, lights) on the attic floor. Critical for mold prevention.

- [ ] Baffles: Install rafter vents in EVERY SINGLE BAY (Full Perimeter) to ensure air washes the roof deck.

- [ ] Install: Blow-in to R-50 depth.

- [ ] Hatch: Insulate and weatherstrip access hatch.

#### Phase 3: The Rebate ($900)

- Requirement: Invoice must clearly state "Total R-50 Installed" and list the square footage.

- Constraint: Must use a CleanBC registered contractor (Knights/Advance are likely registered; check lists).

### Ventilation / air circulation

- that I want to ensure ventilation issues are addressed.  Is there enough air ventilation / air circulation coming from my stucco soffit vents?

- I want the baffles in every bay if possible. Install baffles (e.g., cardboard or foam) at the eaves to maintain airflow from soffit vents to ridge vents, preventing moisture buildup. or at least every second one, to ensure maximum airflow.

- Issues SV-50s vs stucco ventilation.  Currently stucco soffits only 2 vents.  Roofer tried to address this limited air intake with the SV-50 vents but manufacturer sheet says shouldn’t be installed low on roof or as an intake vent, but many companies say that it works fine on the phone menzies product seller even said it was ok. do my stucco soffit vents have issues? Need to verify that circulation happening

- Been told by specialists here the soffit vents are fine .   The upgraded ventilation with 12-air intake vents using SV-50s and exhaust venting using 60’ of 13” Omni vent for adequate venting as required should be sufficient.  Was told this by roofers, insulation installers and EnerTech Solutions Ltd that specialize in testing of these things.

- Only CSC Current Solutions Contracting – talked about 3” band or cutting more ventilation in stucco.

### Sealing

- Sealing and connections

- Batt and weatherstrip access hatch.

- Seal openings/penetrations/Air Sealing:  POLYURETHANE FOAM SEAL ATTIC FLOOR PENETRATIONS INCLUDING OBVIOUS WIRE HOLES, FAN HOUSINGS, PLUMBING AND LIGHT FIXTURES.  TAPE ANY SLIT POLY BACK TOGETHER FOR COMPLETION

- Check insulation on bathroom fans etc. exhausts properly connected insulated

- Confirm the exhaust for fans, solar tube, etc properly connected and insulated

- No matter the insulation, seal all penetrations (lights, pipes, boxes) with spray foam or caulk before insulating.

- Caulk/acoustic sealant around penetrations in the ceiling plane.

- Repair or tape any visible tears in the poly.

- Make sure bath fans and other penetrations are carefully sealed to the vapour barrier.

- seal any and all penetrations from your home leading into the attic.

- Warm moist air is working its way through somewhere.

## Soffit options:

- Cut 4 inch holes in stucco for ventilation pucks

- Cut 3-inch wide slot along the entire length of the soffit (or large rectangular sections between every rafter). They then cover it with a continuous strip of perforated aluminum or vinyl.

## Testing root cause circulation detection:

- identify air leakage detection and determine ventilation air circulation to be addressed prior to installation. Pinpoint issue locations -- address issues before adding more insulation

- approach:  Blower door, pressure on home, smoke tool and thermal imaging to find air leakage and if air circulation is working from soffits? Pulling air in and through the ridge?

- Identify things to be addressed before insulation.

- Talked to Jeffrey Robinson EnerTech Solutions Ltd. 778-425-1420
https://enertech.solutions/  Cost 475+gst

- Related consultation:  Jeffery told me.  Air warmer than inside home.  Trapped condensation on that surface warmer in attic. Attic colder that interior zone coming through.  More ventilation in attic.  More insulation will lower dew point.  too cold in attic dew point.  Issues will be in walls.  Don’t go into the attic to find holes.  Get in attic.  Find all penetrations from home into attic, seal around all penetrations prior to blowing in.  Remove old cellulose.  Fix holes.  Don’t fix too long.   Low slope. Baffles.  Should have baffles. Should put every cavity.  Soffits might need ventilation strip or ventilation pucks puck in soffit 4 inch hole and ventilation puck inlet.  He said I don’t need to add more vents because I have the SV-50s down low for intake.

## Background on SV-50 vents

- https://www.menzies-metal.com/building-supplies/sv50-plastic-roof-vent/

- https://buildersbest.com/literature/Hood%20Roof%20SV50%20Black%20Static%20Roof%20Vent%20V1.pdf

- Product says not to use for intake low on roof, but people at company and all roofers and insulation installers I talked to said it’s ok works normal practice.

- Only Currentsolutions Justin said better to do soffit vents.

- Contacted Nanaimo manager of menzies worked there 20 years .  said this is not an issue.  Call dec 17th 4pm.  https://www.menzies-metal.com/contact-us/ SV-50 + omni ridge vent.

- manager nanaimo 250-888-7994 said it was ok.

## Insulation Quotes

- Knights insulation (corey) they did my crawlspace spray foam insulation.  Was happy with that.  Corey 250-891-5084.  Consultation:  They added ventilation on top of ridge caps.  Don’t need soffits.  Where the placed near ends.  Dams need to ensure no insulation.. If doing removal as well.  Time next week.   https://my.freshbooks.com/?utm_campaign=website&utm_medium=email&utm_source=sendgrid.com#/link/estimate/eyJhbGciOiJIUzI1NiIsInR5cCI6IkpXVCJ9.eyJzeXN0ZW1pZCI6NDQ5MTU0LCJ1c2VyaWQiOjg4OTE1MSwidHlwZSI6ImVzdGltYXRlIiwib2JqZWN0aWQiOjUwMzM1MywiZXhwIjoxNzk3NTQyNDU3LCJyb2xlIjoiY2xpZW50IiwiZW1haWwiOiJyZnJlbW1lckBob3RtYWlsLmNvbSJ9.9NCUcB-iYR9PfUw4rnc8QC1Ec7Rcp41t_p-lEopAgX8?companyName=Corey&entityName=estimate&estimateNumber=27432&ownerEmail=corey%40knightsinsulation.ca&type=primary

- Tomorrow Dec 18.  Advance Insulation Brodie coming tomorrow.  12 SV-50 intake.  They will look at that.

- CSC Current Solutions Contracting Inc. (Justin not available till spring) 
office​@currentsolutions​.info250-514-5245 justin. https://www.currentsolutions.info/services
he said Need new roof vents.  Better soffit vents + baffles.  Pucks.  Ventilation pucks.  I prefer continuous strip band.  Soffit.  Run band to do that.  Strip.  Match ventilation rate.  Masonry cutting blade.  contracted clean bc registered.  Big jobs.  Previously committed.

## Stucco exteriors repairs installs

- Pacific Peak Exteriors.  613-698-2617 stucco soffit. Patrick.  No answer.  Do stucco work.

- Irwin industries.  (250) 382-5154 (no answer)

Control indoor humidity:
In winter, try to keep it around 40% RH depending on how cold it is outside.  running an exhaust fan on low speed 24/7 will assist to remove excess moisture from your home. If you only have older loud bath fans , consider installing an ultra-quiet new bath fan.

## Rebate

- Potential amount upto 900

- Best Practice Guide for Air Sealing and Insulation Retrofits. The Leverage: This guide explicitly mandates that ventilation must be maintained. If they block your vents, they are technically violating the program rules that allow them to offer the rebate.