# Technical Claim Brief: 1517 San Juan Ave

Date: December 24, 2025 Claim Reference: [Insert BCAA Claim Number] Subject: Systemic Ventilation Failure and Resulting Structural Moisture/Mold Damage

## 1. Executive Summary

This claim pertains to widespread moisture ingress and biological growth (black mold) discovered on ceiling joists and within wall-to-ceiling junctions. This damage is the direct "resulting damage" of a non-compliant roof ventilation system installed in 2024 by BCAA-recommended vendor, Vintage Roofing Ltd.. The system has created a "massive vacuum" in the attic, pulling warm, moist air from the living space through the building envelope, where it condenses on the cold side of the vapor barrier.

## 2. Evidence of Technical Non-Conformance

The installation deviates from both manufacturer specifications and the original contractual scope of work:

- Manufacturer Violation (SV-50 Vents): The roofer installed 12 SV-50 vents as air intake. The manufacturer’s technical data (Menzies) and QAI Listing B1024-2 explicitly state: "Do not use as intake vent" and require a minimum 3.5:12 pitch.

- Unauthorized Scope Increase: The signed quote (#203) specified 8 intake vents. The contractor installed 12 vents without a written change order, increasing the number of unauthorized roof penetrations by 50%.

- Knowledge of Risk: On Dec 23, 2025, the vendor admitted they were aware the manufacturer does not approve these vents for intake but installed them regardless, citing "liability" concerns for the manufacturer rather than building code compliance.

## 3. The "Massive Vacuum" Physics

The attic is currently a physically imbalanced system:

- Exhaust Capacity: 60’ of Omni Ridge Vent provides approx. 1,080 sq. in. of exhaust.

- Intake Capacity: 12 SV-50 vents provide approx. 600 sq. in. of intake (assuming they function as intended).

- Deficit: A ~480 sq. in. intake shortage creates a constant negative pressure (vacuum) that sucks household moisture into the attic space.

## 4. Scope of Resulting Damage

The damage is not localized but systemic, affecting the building envelope integrity:

- Biological Growth: Shadowing and black mold are appearing on ceiling joists.

- Vapor Barrier Compromise: Moisture is trapped under the poly vapor barrier.

- Migration: Because the vapor barrier connects to exterior walls, moisture is migrating into wall cavities, threatening the structural wood behind the stucco.

## 5. Required Remediation (The "Dirty Job")

A "patch and paint" approach is insufficient. To return the home to a pre-loss state and prevent immediate recurrence, the following scope is required:

- Full Insulation Removal: Vacuuming out all existing cellulose to prevent mold spore spread during drywall work.

- Structural Remediation: Professional antimicrobial treatment of all ceiling joists and wall top plates.

- Envelope Sealing: Polyurethane foam sealing of ALL attic floor penetrations (wires, fans, lights) while the ceiling is open to stop the moisture source.

- Drywall Replacement: Removal of affected ceiling and wall drywall to access and repair the vapor-barrier-to-wall connections.

# Script for the BCAA Adjuster

Use these talking points when speaking with BCAA or the adjuster to ensure the focus stays on "Resulting Damage" from their "Recommended Vendor."

The "Preferred Vendor" Angle:

"BCAA recommended Vintage Roofing for my 2024 repair. I trusted their professional expertise, but I have since discovered they installed a ventilation system that explicitly violates the manufacturer's technical specifications. They used exhaust vents as intake, which has created a massive vacuum in my attic."

The "Resulting Damage" Argument:

"This isn't just about a 'bad roof.' This is about the resulting damage inside my home. The ventilation imbalance is pulling moisture through my ceilings, causing mold on my joists and trapping water under my vapor barrier. Because the barrier connects to my exterior walls, I am concerned about hidden rot in the wall cavities."

The "Dirty Job" Necessity:

"I need to ensure the restoration is done right. We can't just paint over the mold. The insulation needs to be vacuumed out so the joists can be treated, and the ceiling needs to be opened up so we can seal the air leaks and the vapor barrier properly. Otherwise, the new drywall will just mold again in six months."

The Admission of Fault:

"The roofer actually admitted to me yesterday that they knew the manufacturer doesn't approve these vents for intake, but they did it anyway because 'roofers do it all the time.' That 'standard practice' has caused structural damage to my house, and since BCAA recommended them, I’m looking to you to help make this right."

Would you like me to help you draft a specific email to the Restoration Company (like KCC) to make sure they include the "vapor barrier to wall" inspection in their initial report?