# SITE VISIT CHECKLIST: Justin (Current Solutions)
PRE-CALCULATED NFA ANALYSIS

| Component | Source | NFA |
| --- | --- | --- |
| EXHAUST: Lomanco OR-4 Omni Ridge | Invoice: "60' of 13" Omni vent" | 60 × 18 = 1,080 sq. in. |
| INTAKE: Menzies SV-50 vents | Invoice: "8-air intake vents" / Site count: 12 | 12 × 50 = 600 sq. in. |
| INTAKE: Soffit vents | Photos: 4 small louvered (E/W only) | 4 × 25 = ~100 sq. in. |
| TOTAL INTAKE |  | ~700 sq. in. |
| DEFICIT |  | 380 sq. in. (35%) |

Conclusion: System is imbalanced. Exhaust exceeds intake by 35%, creating negative pressure.

PART 1:  WALKTHROUGH

- [ ] 1. The Engine (Roof Peak)

  - Point to the 60-foot Omni Ridge Vent

  - Say: "Per the invoice, that's 60 linear feet at 18 NFA per foot = 1,080 sq. in. of exhaust."

- [ ] 2. The Choke Point (Lower Roof)

  - Point to the 12 SV-50 Vents

  - Say: "The invoice called these 'air intake vents' but Menzies rates them 'exhaust only — not to be used as intake.' At 50 NFA each, that's only 600 sq. in. — already a deficit before we even count the missing soffit intake."

  - Show him the Menzies SV-50 product page

- [ ] 3. The Evidence (Inside)

  - Show the Ceiling Ghosting (joist shadowing visible in hallway/bedroom)

  - Show the Attic Condensation photo

  - Say: "This appeared after the April 2024 roof install. We treated the worst mold spots with Zinsser on Jan 1, 2026."

- [ ] 4. The Dead Zone (Soffits)

  - Show the Stucco Soffits — only 4 tiny louvered vents (East/West)

  - Point out North/South have ZERO soffit vents

  - Say: "With intake only at mid-roof and minimal at eaves, the bottom 4 feet of deck has no airflow — a dead zone."

PART 2: THE CRITICAL QUESTIONS

Ask these to get the "Expert Verdict" you need:
Q1. The "Vacuum" Confirmation
"With 1,080 sq. in. exhaust and only 700 sq. in. intake, that's a 35% deficit. Is this creating negative pressure that pulls moist air from my house into the attic?"
Q2. The "Short Circuit" Check
"If I install continuous soffit vents, will the SV-50s start acting as exhaust and short-circuit the airflow? Do they need to be removed?"
Q3. The "Code Trap" (BCBC 9.19.1.3)
"The code requires 63mm airway at the wall plate. Since the SV-50s are 4 feet up the roof, isn't the eave area a dead zone? Doesn't that violate 9.19.1.3?"
Q4. The "Informed Consent" Question
"If Mark had disclosed that SV-50s are rated 'exhaust only' and offered proper soffit venting as an alternative, I could have made an informed choice. Is that something a professional should have disclosed?"

PART 3: THE "DELIVERABLE" (The Ask)

- [ ] Get Written Confirmation

  - Ask for an email stating that intake needs to move to the soffits to balance the system and meet code

- [ ] Verify the NFA Calculations

  - Ask him to confirm the math: 1,080 exhaust vs. 700 intake = 35% deficit

### KEY SOURCES (For Reference)

| Source | Key Finding |
| --- | --- |
| Vintage Roofing Invoice | "8-air intake vents using SV-50" / "60' of 13" Omni vent" |
| Menzies SV-50 Specs | "Exhaust vent... Not to be used as an intake vent" |
| Lomanco OR-4 Specs | 18 sq. in. NFA per linear foot |
| BC Building Code 9.19.1.3 | Requires 63mm airway at wall plate (eaves) |
| Lstiburek (2022) | "Wash the entire underside of the roof deck" |

Calculations based on invoice data, manufacturer specifications, and site observations compiled Jan 16, 2026.