# Roofing and ventilation questions and things to confirm

Context:  low-slope roof rancher, new roof installed, ridge vent extra vents added on roof as well, issue might be bottom, seeing on joists appearances start of black mold, looked spots saw moisture under vapor barrier.

Testing

- Help with air leakage detection and determine ventilation air circulation

- Blower door, pressure on home, smoke tool and thermal imaging to find air leakage and if air circulation is working from soffits? Pulling air in and through the ridge?

- Fan insulation correct make sure connection insulated properly

- Identify things to be addressed before insulation.

- Pinpoint issue locations e.g. kids room back corner

- Address issues before adding more insulation

- Jeffrey Robinson EnerTech Solutions Ltd. 778-425-1420
https://enertech.solutions/

- Cost 475+gst

Was in junk.

Air warmer than inside home.  Trapped condensation on that surface warmer in attic.

Attic colder that interior zone coming through.  More ventilation in attic.  More insulation will lower dew point.  too cold in attic dew point.  Issues will be in walls.  Don’t go into the attic to find holes.

Get in attic.  Find all penetrations from home into attic, seal around all penetrations prior to blowing in.

Remove old cellulose.  Fix holes.  Don’t fix too long.

Low slope. Baffles.  Should have baffles. Should put every cavity.

Soffits might need ventilation puck in soffit 4 inch hole and ventilation puck inlet.

Ridge vent.

Issues with

- Remove existing celllose / vacuum

- Baffles / soffit chutes a must

- Seal openings/penetrations

- Check insulation on bathroom fans etc. exhausts properly connected insulated

- 4 inch soffit ventilation pucks only 2 soffit vents each side.

Colby Pitts (original estimate)
250-510-9070
colby@knightsinsulation.ca

Correy

250-891-5084

Ventilation / air circulation

- Is there enough air ventilation / air circulation coming from my stucco soffit vents?

- do my stucco soffit vents have issues? Need to verify that circulation happening

Baffles (soffit chutes) and circulation

Install baffles (e.g., cardboard or foam) at the eaves to maintain airflow from soffit vents to ridge vents, preventing moisture buildup.

Sealing and connections

- Confirm the exhaust for fans, solar tube, etc properly connected and insulated

- No matter the insulation, seal all penetrations (lights, pipes, boxes) with spray foam or caulk before insulating.

Insulation and R-value:

- R-Value: target r-value? 50?.

- Types:  Application: Use rigid boards on the roof deck and blown-in/batts on the attic floor, or a hybrid system for maximum performance

- Fan insulation correct make sure connection insulated properly.

- Will remove old cellulose

Rebate

- Potential amount